DESIGN DOCUMENT

ON

ARTIFICIAL INTELLIGENCE ENABLED LOCO BASED

CONDITION MONITORING SYSTEM

(AICMS)

FOR INDIAN RAILWAYS

MANUFACTURED BY

DUCTUS INFODOT PVT. LTD.

OFFICE ADDRESS:

S-64, BASEMENT, GREATER KAILASH PART-1, NEW DELHI-110048

# DOCUMENT REVISION HISTORY

| Rev. | Date | Description of Change | Prepared By | Checked By | Approved By |
| --- | --- | --- | --- | --- | --- |

# APPENDIX — ABBREVIATIONS & ACRONYMS

| S.No. | Term / Acronym | Explanatory Description |
| --- | --- | --- |
| 1 | AICMS | Artificial Intelligence Enabled Loco Based Condition Monitoring System |
| 2 | OHE | Over Head Equipment |
| 3 | AI / ML | Artificial Intelligence / Machine Learning |
| 4 | RDSO | Research Designs & Standards Organisation |
| 5 | PLW | Patiala Locomotive Works |
| 6 | IR | Indian Railways |
| 7 | PCEE | Principal Chief Electrical Engineer |
| 8 | OCC | Operational Control Centre |
| 9 | DCC | Divisional Control Centre |
| 10 | GPS | Global Positioning System |
| 11 | IRNSS | Indian Regional Navigation Satellite System |
| 12 | IP68 | Ingress Protection Rating — fully dustproof & waterproof |
| 13 | EMI / EMC | Electromagnetic Interference / Electromagnetic Compatibility |
| 14 | CRIS | Centre for Railway Information Systems |
| 15 | TDMS | Traction Data Management System |
| 16 | ADD | Automatic Dropping Device (for pantograph) |
| 17 | BOM | Bill of Materials |
| 18 | DLP | Defects Liability Period |
| 19 | OEM | Original Equipment Manufacturer |
| 20 | API / SDK | Application Programming Interface / Software Development Kit |
| 21 | STQC | Standardisation Testing and Quality Certification Directorate |
| 22 | OWASP | Open Web Application Security Project |
| 23 | NDA | Non-Disclosure Agreement |
| 24 | PSU | Power Supply Unit |
| 25 | HMI | Human Machine Interface |
| 26 | TOC | Train Operations Controller |
| 27 | FAT | Factory Acceptance Test |
| 28 | SAT | Site Acceptance Test |
| 29 | NAS | Network Attached Storage |
| 30 | GPU | Graphics Processing Unit |
| 31 | FPS | Frames Per Second |
| 32 | SOD | Schedule of Dimensions |
| 33 | CII | Critical Information Infrastructure |
| 34 | MeitY | Ministry of Electronics & Information Technology |

# APPLICABLE STANDARDS & REFERENCES

| S.No. | Applicability of the Standard for AICMS System | Standard / Reference |
| --- | --- | --- |
| 1. | Railway EMC — Emission & Immunity of signalling and telecommunications apparatus | EN 50121-4 |
| 2. | Environmental Conditions for equipment — Signalling and Telecommunications | EN 50125-3 |
| 3. | Information Technology Equipment Safety | BIS (IS-13252: Part 1: 2010 or latest) |
| 4. | Environmental Testing — Mould Growth Test of PCB | IEC 60068-2-10 or latest |
| 5. | Electromagnetic signal interference sensitivity for all OHE systems | EN 50121-3-2 |
| 6. | Vibration and shock tests for loco-mounted equipment | IEC 61373 & IEC 60571 |
| 7. | Fire Retardant STP CAT-6 cable for network connectivity | EIA/TIA-568-B.2-1 & IEC/EN 60332-1 |
| 8. | Electrical Power Cable for Rolling Stock — RDSO Approved Sources | RDSO ELRS/SPEC/ELC/0019, Rev. 3 Feb. 2017 (or latest) |
| 9. | Cybersecurity Guidelines for CII of Indian Railways — SCADA System Compliance | TI/IN/0052(02/25) or latest |
| 10. | Network Video Interface — Camera System Integration | ONVIF Profile 'S' |
| 11. | Camera and mounting dimensions on locomotive roof | SOD-2022 or latest |

# INDEX

| Section | Description | Page Ref. |
| --- | --- | --- |
| A. | INTRODUCTION — Background, Objectives, System Architecture, Parameters | 5–14 |
|  | Parameter Range / Accuracy Table | 5–7 |
|  | General Layout of System (System Architecture Diagram) | 8–10 |
|  | Overall System Architecture Description | 11–14 |
| B. | SUBSYSTEM COMPONENTS DESCRIPTION / SCOPE | 15–42 |
|  | B1: Camera & Vision System (OHE Video Detection Unit) | 15–19 |
|  | B2: AI / Edge Computing Unit (On-Board Intelligence) | 20–24 |
|  | B3: GPS / IRNSS Location & Synchronisation Module | 25–27 |
|  | B4: Communication & Networking Module | 28–31 |
|  | B5: On-Train Data Storage Unit | 32–34 |
|  | B6: Central Server & Cloud Data Processing | 35–42 |
| C. | DASHBOARD FOR REPORT GENERATION & ALARM MANAGEMENT | 43–47 |
| D. | BILL OF MATERIALS (BOM) | 48–52 |
| E. | POWER SUPPLY & REDUNDANCY | 53–60 |
| F. | OPERATION & MAINTENANCE OVERVIEW | 61–64 |
| G. | SYSTEM TRIALS & ACCEPTANCE | 65–67 |
| H. | CONCLUSION | 68 |
| I. | ADDITIONAL INFORMATION / ANNEXURES | 69–80 |

A) INTRODUCTION

## A.1 Background & Purpose

The inspection of Over Head Equipment (OHE) parameters on Indian Railways has traditionally been conducted through periodic foot patrolling and tower car inspections. While these methods have served Indian Railways for decades, they are inherently limited by inspection frequency, human capacity, and inability to detect transient defects occurring at full operational speeds. Such gaps between defect occurrence and detection allow minor faults to escalate into major failures, causing operational disruptions and safety hazards.

The purpose of development of the Artificial Intelligence Enabled Loco Based Condition Monitoring System (AICMS) is to shift Indian Railways from Periodic Maintenance to Predictive Maintenance. AICMS is an automated detection system for Online Condition Monitoring of Overhead Equipment (OHE) and OHE-Pantograph Interaction for real-time detection of defects and generation of necessary alarms.

Ductus Infodot Pvt. Ltd., with their OEM, has prepared this Design Document in accordance with RDSO Instruction No. TI/IN/0057 issued by the Traction Installation Directorate, Research Designs & Standards Organisation (RDSO), Lucknow. This document defines the complete design framework of the AICMS system for deployment on railway-manufactured electric locomotives.

## A.2 System Objectives

- Predictive Maintenance: Shift from scheduled periodic OHE inspections to an AI-driven predictive maintenance model across all electrified sections.

- Continuous Monitoring: Deploy a loco-mounted system to continuously monitor OHE-pantograph interaction at speeds up to 160 kmph in all weather and lighting conditions.

- Automated Detection: Utilize AI/ML models to automatically detect, classify, and report infrastructure defects without manual intervention.

- Failure Reduction: Minimize OHE-related failures, disruptions, and accidents through early detection and timely corrective maintenance.

- Real-Time Alerts: Transmit instantaneous, GPS-tagged alarms, defect images, and reports directly to railway control centers.

- Trend Analysis: Build a comprehensive, searchable historical database of OHE conditions for long-term infrastructure management.

## A.3 Parameters & Detectable Events

The AICMS system shall be capable of monitoring, detecting, and reporting the following OHE and Pantograph defects and parameters as specified in RDSO TI/IN/0057:

| S.No. | Defect / Parameter to be Detected | Category | Detection Method |
| --- | --- | --- | --- |
| 1 | Arcing — Detection & Severity Classification | OHE Defect | Camera + UV sensor + AI Model |
| 2 | Faulty Dropper Wires | OHE Defect | Camera + AI Model |
| 3 | Misaligned Section Insulator / Short Neutral Section | OHE Defect | Camera + AI Model |
| 4 | Broken Branches / Foreign Objects (kites, wires, etc.) | OHE Defect | Camera + AI Model |
| 5 | Hitting of Register Arm by Pantograph | OHE Defect | Camera / Impact Sensor |
| 6 | Hard Spots — Register Arm Lacking Vertical Movement | OHE Defect | Vibration Sensor + AI |
| 7 | Converging Wires — Height Change causing Pantograph Impact | OHE Defect | Camera + AI Model |
| 8 | Catenary Strands Cut | OHE Defect | Camera + AI Model |
| 9 | Bird Nests on OHE Structures | OHE Defect | Camera + AI Model |
| 10 | Proximity of Vegetation to OHE | OHE Defect | Camera + AI Model |
| 11 | Pantograph Flip (Abnormal Pantograph Behaviour) | Pantograph Defect | Camera + AI Model |
| 12 | Ride Quality & Sway (Vibration / Oscillation) | Pantograph Defect | Camera + AI Model |
| 13 | Real-time Geo-location (GPS/IRNSS) of detected event | Location Data | GPS/IRNSS Module |
| 14 | Operational Speed at point of defect detection | Speed Data | GPS |

## A.4 Defect Severity Classification

All detected defects shall be automatically classified into three severity levels by the on-board AI system to enable prioritized response:

| Severity Level | Classification | Trigger Action | Response Time |
| --- | --- | --- | --- |
| Severity-I | Critical — Immediate Attention Required | Audio-visual alarm at control centre + SMS alert with GPS link to main railway server. | Immediate |
| Severity-II | Urgent — Earliest Attention Required | Event logged + Alert transmitted to control centre for priority maintenance scheduling | Within shift |
| Severity-III | Monitor — Planned Action Required | Event recorded in database for trend analysis and planned maintenance intervention | Planned cycle |

## A.5 General Layout of AICMS System — Architecture Overview

The AICMS system follows a three-tier architecture consisting of: (1) On-Train Loco-Mounted System, (2) Communication & Networking Layer, and (3) Central Server / Cloud & Control Centre. The following describes the overall system layout:

AICMS — OVERALL SYSTEM ARCHITECTURE

TIER 1: ON-TRAIN (LOCO-MOUNTED) SYSTEM [ Camera Vision Unit (1080p @ 160fps) ] + [ AI/Edge GPU Computing Unit ] + [ GPS/IRNSS Module ] + [ On-Board NAS Storage (min. 1 month) ] + [ PSU: 72V & 110V DC/110V AC ]

▼ Wireless Network (4G/5G/LTE — Avg. < 20 sec latency; P95 < 90 sec) ▼

TIER 2: COMMUNICATION & NETWORKING LAYER [ GSM Wireless Module ] ↔ [ Network Fault-Tolerant Buffer ] ↔ [ API Gateway ]

▼ Real-time Data & Alarm Transmission to Control Centres ▼

TIER 3: CENTRAL SERVER / CLOUD & CONTROL CENTRES [ Central Web Server + Dashboard ] ¦ [ MeghRaj/CRIS Cloud Storage ] [ OCC / DCC Alarm Terminals ] ¦ [ SMS Gateway ] ¦ [ TDMS / CRIS Integration via API ]

Figure A.5.1 — AICMS Brief System Schematic (Loco-Mounted Hardware Layout)

Schematic showing Above-Roof (Camera Unit, GSM/GPS/Bluetooth Antennas) and Under-Roof (IO Controller, Power Converter, UPS) components, Pantograph-mounted Accelerometer Sensor, and connectivity to IR Cloud Server and OEM Remote Access.

B) SUBSYSTEM COMPONENTS DESCRIPTION / SCOPE

## B1:  Camera & Vision System (OHE Video Detection Unit)

### B1.1 Overview & Purpose

The Camera System is the primary sensing unit of the AICMS system. It is mounted on the locomotive roof within the limits specified in SOD-2022 (or latest) with IP68 protection (ensuring the equipment is completely dustproof and waterproof against high-pressure water jets), and is responsible for capturing continuous high-speed video footage of the OHE infrastructure, contact wire, dropper wires, section insulators, and pantograph interaction at full operational line speeds up to 160 kmph.

### B1.2 Technical Specifications

| Parameter | Specification / Requirement |
| --- | --- |
| Camera Resolution | Minimum 1080p (Full HD) |
| Frame Rate | Minimum 160 Frames Per Second (FPS) to capture transient defects at operational speed |
| Lens Type | Self-cleaning, scratch-resistant against carbon dust and debris |
| Operating Temperature | -10°C to +50°C Ambient; up to 70°C metallic surface temperature under sun |
| Camera Count | Minimum 2 units per locomotive (forward-facing + pantograph-facing) — final configuration as per OEM design validated against SOD-2022 Field of View requirements |
| Field of View | Minimum 60° horizontal — sufficient coverage of OHE contact zone and pantograph crown as per SOD-2022 |
| Night / Low-light | Active illumination |
| Video Pre/Post Trigger | 5 seconds pre-trigger + 5 seconds post-trigger video capture on defect detection |
| Lens Material | Non-deformable materials for dimensional stability across temperature extremes |
| Camera Interface | GigE Vision / PoE (IEEE 802.3af/at) or USB 3.0 — as per OEM camera module design |
| Mounting | Within SOD-2022 limits; provision for wireless pantograph-mounted equipment |
| EMI/EMC Compliance | EN 50121-4, EN 50121-3-2 — protected against OHE electromagnetic interference |
| Vibration Resistance | IEC 61373 — designed to withstand all operational vibration and shock conditions |

### B1.3 Functional Requirements

- Continuously capture OHE and pantograph interaction footage at full train operating speed without any operator intervention.

- Self-cleaning optics to ensure maintenance-free operation during the warranty period — no manual cleaning or recalibration required.

- All defect scan images and videos to be recorded and stored on the on-board NAS and uploaded to the central server.

- The camera system utilizes temperature-stable materials to ensure consistent measurement accuracy under all environmental conditions specified in Clause 3.0 of RDSO Instruction No. TI/IN/0057.

- Full details regarding the system's working principles, sensor resolution, frame rates, GPU specifications, local storage, and boundary conditions are included in this document.

## B2:  AI / Edge Computing Unit (On-Board Intelligence)

### B2.1 Overview & Purpose

The AI/Edge Computing Unit is the intelligence core of the on-train system. It receives raw video and sensor data from the Camera System, IMU, GPS, and other sensors in real time, processes this data using AI/ML models on-board, classifies detected defects by severity, triggers alarms, and initiates data transmission — all without requiring any client or operator intervention.

### B2.2 Technical Specifications

| Parameter | Specification / Requirement |
| --- | --- |
| Processing Architecture | Dedicated GPU/NPU-based edge AI inference unit |
| AI Framework | Deep Learning based Computer Vision (CNN/Transformer architecture or equivalent) |
| Defect Detection Accuracy | AI model to reliably detect all events in Clause 4.0 of TI/IN/0057 without fail |
| False Alarm Rate | System preferably shall not generate false alarms (limitations to be declared by OEM) |
| Processing Latency | Real-time, on-board edge analytics to reduce reporting latency |
| Time Synchronisation | Video and sensor data time-aligned within ±50 milliseconds |
| Operating Temperature | -10°C to +50°C |
| Enclosure Protection | IP54 minimum — for internal locomotive cabinet installation (under-roof) |
| Local Storage | NAS with capacity for minimum 1 month of full data retention (accessible remotely) |
| Self-Diagnostics | System capable of self-diagnosis of failure/breakdown/theft/vandalism; alarm issued to IR in real time |
| User-Defined Rules | System shall allow setup of user-defined Alarm Rules |
| ADD Integration | System shall be ADD-ready — capable of interfacing with Automatic Dropping Device |

### B2.3 Severity Analysis & Alarm Logic

On detection of any defect, the on-board AI model automatically classifies the event into one of three severity levels based on pre-defined and user-configurable alarm rules:

- Severity-I (Critical): Triggers immediate, continuous audio-visual alarms at the designated control center and generates automated SMS alerts featuring a real-time Google Maps location link and a direct report link. The audible alarm continues until actively acknowledged by control personnel.

- Severity-II (Urgent): Logs the event in the database and transmits an alert to the control center for priority maintenance scheduling within the current operational shift.

- Severity-III (Monitor): Records the event in the central database for long-term trend analysis, asset tracking, and planned maintenance cycles.

All generated reports comprehensively include a high-resolution photograph or video clip of the defect, precise GPS coordinates, date, timestamp, and defect classification. Furthermore, the system maintains a secure, unalterable log tracking all alarms, events, operator interventions, and real-time system diagnostics

## B3:  GPS / IRNSS Location & Synchronisation Module

### B3.1 Overview & Purpose

The GPS/IRNSS Module provides precise real-time geo-location tagging of all detected defects and system events. This enables Indian Railways to pinpoint the exact location of every OHE defect on the route map, facilitating rapid field response, trend mapping, and infrastructure management.

### B3.2 Technical Specifications

| Parameter | Specification / Requirement |
| --- | --- |
| Navigation System | GPS and IRNSS (Indian Regional Navigation Satellite System) dual-mode |
| Position Accuracy | GPS/IRNSS location accuracy ≤ 5 m CEP (Circular Error Probable) under open-sky conditions — suitable for OHE defect geo-tagging |
| Time Synchronisation | System date and timestamp synchronised with GPS/IRNSS time standard |
| Location Reporting | GPS coordinates attached to all alarm, defect, and event reports |
| Data Integration | GPS data integrated with video/sensor data stream for synchronized reporting |
| Operating Environment | All-weather, all-season performance across Indian Railway network |
| Altitude Range | Up to 2500m above mean sea level |
| Anti-jamming | Protected against electromagnetic interference from OHE (25 kV AC environment) |

## B4:  Communication & Networking Module

### B4.1 Overview & Purpose

The Communication & Networking Module handles all wireless data transfer between the on-train AICMS system and the central server / cloud infrastructure. It ensures that defect data, alarms, and reports are transmitted to railway servers in near real-time and that fault-tolerant buffering is available during poor network coverage zones.

### B4.2 Technical Specifications

| Parameter | Specification / Requirement |
| --- | --- |
| Communication Technology | GSM; fallback via available mobile network |
| Defect-to-Server Latency | Average < 20 seconds; 95th percentile < 90 seconds (subject to connectivity) |
| Fault-Tolerant Buffering | If network is unavailable, system buffers data and auto-transmits on reconnection |
| Data Destination | Railway CRIS server via wireless network (API-based) |
| Network Security | End-to-end encrypted data transmission; compliant with CII cybersecurity guidelines TI/IN/0052(02/25) |
| Web-Server Protocol | Internet-based web server; multi-user password protected; differential access rights |
| Data Export | MS-Excel and XML format export; on-demand software-based data transfer |
| API/SDK | API/SDK shared with purchaser / CRIS for integration |
| Cable Type | STP CAT-6 or better — EIA/TIA-568-B.2-1 and IEC/EN 60332-1 compliant |
| User Access | Unlimited read-only users (TOCs, Infra Managers, OEMs) with instantaneous alarm access |

### B4.3 Alarm & Alert Delivery

- Audio-visual alarms delivered at designated control location with GPS coordinates and defect photograph/video.

- SMS alerts generated for Severity-I events with location Google link and report link.

- Alarm signal for system failure/vandalism/theft is distinct from OHE defect alarm signals.

- Audio alarm continues until acknowledged — no silent dismissal permitted.

B4.4 Cybersecurity Vulnerability Assessment & Compliance Audit

In compliance with RDSO Cybersecurity Guidelines for Critical Information Infrastructure (CII) of Indian Railways (TI/IN/0052(02/25) or latest), the AICMS system shall be subjected to a comprehensive Cybersecurity Vulnerability Assessment and Compliance Audit prior to commissioning and at periodic intervals during the Defects Liability Period (DLP). This assessment shall be conducted by a STQC-empanelled or MeitY-approved agency.

Scope of Cybersecurity Assessment

- On-Train AICMS unit — including AI/Edge Computing Unit, Communication Module, on-board NAS, and all software interfaces.

- Central Web Server, Dashboard Application, API Gateway, and Cloud Infrastructure (MeghRaj/CRIS platform).

- All wireless communication channels including GSM uplink and OEM remote access interfaces.

- User authentication mechanisms, access control framework, and data encryption protocols.

#### Technical Scoping & Infrastructure Parameters

To facilitate targeted penetration testing (VAPT), the system validation metrics are explicitly defined as follows:

- Core Infrastructure Audited (Per Unit Configuration): 1 Linux/Unix Server, 1 Network Router, 1 Web Server, 1 Database, and 1 Public-facing IP address.

- Application & API Architecture: 1 Web Application interface and 1 REST-based API layer supporting 3 distinct user access roles (Administrator, Engineer, and Read-Only Operator).

- Functional Validations: File Upload functionality is active and requires security auditing, alongside a complete Source Code Review (Line of Code validation) conducted in a Test / UAT environment.

- Audit Exclusions: Local desktops/laptops, corporate email systems, domain controllers, native mobile applications (Android/iOS), and third-party e-commerce/payment gateway software are completely out of scope.

Compliance Requirements

- Assessment methodology shall conform to OWASP guidelines and CERT-In prescribed frameworks.

- OEM shall submit a Cybersecurity Compliance Certificate from a STQC-empanelled agency prior to FAT and again prior to SAT.

- Any vulnerabilities identified during the assessment shall be remediated by the OEM within the timeframe stipulated by Indian Railways, and a re-assessment report submitted before operational deployment.

- Periodic re-assessment shall be conducted at intervals not exceeding 12 months or after any major software update, whichever is earlier, throughout the DLP.

## B5:  On-Train Data Storage Unit (NAS)

### B5.1 Overview & Purpose

The On-Train Data Storage Unit (NAS — Network Attached Storage) provides resilient, high-capacity local storage on the locomotive for buffering all raw video, sensor data, AI-processed results, alarms, and GPS-tagged event records generated during operation.

### B5.2 Technical Specifications

| Parameter | Specification / Requirement |
| --- | --- |
| Minimum Local Storage | Sufficient for complete data storage / backup for at least 1 month of continuous operation |
| Remote Access | Data accessible by remote means as required by Indian Railways |
| Data Format | Reproducible electronic format; exportable to PDF, CSV, Excel, XML |
| Data Resilience | RAID or equivalent redundancy to prevent data loss under vibration/shock conditions |
| Operating Conditions | Compliant with IEC 61373 for vibration; IP54 minimum enclosure protection (under-roof internal installation) |
| File Management | Automatic circular overwrite after minimum retention period if not uploaded |
| Defect Records | All scan images and videos of detected defects stored locally and on server |
| System Log | Alarms, events, operator log maintained; searchable by location, date, time, event type |

## B6:  Central Server & Cloud Data Processing

### B6.1 Overview & Purpose

The Central Server and Cloud Infrastructure form the core data management foundation of the AICMS ecosystem. Field telemetry and video feeds from the locomotive-mounted systems are aggregated, stored, and processed dynamically. Authorized railway personnel can access this data through a secure, web-based dashboard that provides real-time trend analysis, historical health comparisons, predictive maintenance insights, and centralized alarm management workflows.

### B6.2 Central Server — Technical Specifications

| Parameter | Specification / Requirement |
| --- | --- |
| Server Type | Internet-based Web Server for exception report generation and data management |
| Cloud Platform | As per MeghRaj initiative of MeitY / CRIS cloud — Government-approved cloud |
| Data Retention — Processed | Processed data and analysis reports (including alarms) stored for minimum 1 year |
| User Access | Unlimited read-only users — TOCs, Infra Managers, OEMs with instantaneous access |
| Access Levels | Multiple levels: Write-only, Read-only, Administrator |
| Authentication | Multiple-user password-protected login; access via public internet with credentials |
| Data Export | MS-Excel and XML format; on-demand software-based data transfer to other railway applications |
| Integration | API/SDK shared with CRIS |
| Zonal Control Centre | Controlling Zonal Railway to establish control centre for real-time alert/report distribution to ZR/Division |
| Data Ownership | All AICMS data is the property of Indian Railways; no sharing with third party without prior IR permission |
| Data Localization | Data compiled, stored and transferred as per Government of India Data Localization rules |

### B6.3 Data Viewer System Capabilities

- Manage all field data and make available for advanced trend analysis to authorized users.

- Filter data by location, date, time, event type to allow operator search across entire database.

- View and compare every measurement against previous measurements; predict parameters approaching tolerance limits.

- Generate and export reports in PDF, CSV, Excel, XML formats as required.

- Historical measurement value to be analysed with predictive data flagging when parameters approach tolerance boundary.

C) DASHBOARD FOR REPORT GENERATION & ALARM MANAGEMENT

## C.1 Monitoring & Measuring Software

The AICMS Dashboard provides a centralized, web-based Human Machine Interface (HMI) for real-time monitoring, alarm management, data visualization, and report generation. It is accessible to authorized users across engineering and maintenance teams via secure internet connection.

## C.2 Dashboard Features

| Feature | Description |
| --- | --- |
| Real-time System View | Live dashboard showing current status of all AICMS-equipped locos, active alarms, and recent events |
| Alarm Management | Audio-visual alarm display; alarm acknowledgment workflow; alarm history with GPS and media |
| Defect Report Viewer | View photograph/video of each defect event with location, date, time, defect type, severity level |
| SMS Alert System | Automatic SMS generation for Severity-I events with Google location link and report link |
| Trend Analysis | Historical comparison of OHE parameters; graphical trend charts; predictive flagging |
| Search & Filter | Search defects by location, route section, date range, event type, severity level |
| Geo-location Map | Map view showing GPS-tagged defect locations across the IR network |
| Report Generation | Generate, export and download reports in PDF, CSV, Excel, XML formats |
| User Management | Role-based access: Administrator, Read-Write, Read-Only for TOCs, Infra Managers, OEMs |
| System Health Monitor | Self-diagnostic status of AICMS equipment on each loco; failure/vandalism alerts |
| Multi-user Access | Unlimited simultaneous read-only users with no performance degradation |

## C.3 Report Types Generated

- Exception Reports — Defects detected, classified by severity, with media and GPS data.

- Daily / Weekly / Monthly Summary Reports — Aggregated OHE health statistics per route section.

- Trend Analysis Reports — Historical comparison of OHE condition over time for predictive maintenance.

- Alarm & Event Logs — Complete time-stamped log of all alarms, events, and operator actions.

- System Health Reports — Equipment status, uptime, and diagnostic information for each AICMS-fitted loco.

- Custom Reports — Operator-defined reports by route, loco, date, severity, or defect type.

D) BILL OF MATERIALS (BOM)

## D.1 Loco-Mounted Hardware BOM (MSD should be minimised)

| S.No. | Item Description | Sub-system | Qty (Per Loco) |
| --- | --- | --- | --- |
| 1 | High-Speed Camera Unit (1080p @ 160fps, Self-cleaning) | Camera Vision System | 2 |
| 2 | Illumination Unit for Night Vision | Camera Vision System | 1 |
| 3 | Camera Mounting Bracket (SOD-2022 compliant) | Camera Vision System | As req. |
| 4 | AI/Edge Computing Unit with GPU/NPU | AI Edge Unit | 1 |
| 5 | Accelerometer / IMU Sensor (Pantograph-mounted, wireless) | Vibration / Condition Sensing | 1 |
| 6 | GPS / IRNSS Dual-Mode Navigation Module | Location Module | 1 |
| 7 | On-Board NAS Storage Unit (min. 1-month capacity) | Data Storage | 1 |
| 8 | GSM / Latest Wireless Communication Module | Communication | 1 |
| 9 | Roof Interconnection / Junction Unit | Cabling/Connectivity | 1 |
| 10 | Power Supply Unit (72V & 110V DC / 110V AC) | Power Supply | 1 |
| 11 | Earthing & Surge Protection Assembly | Power Supply | 1 set |
| 12 | STP CAT-6 Network Cable (EIA/TIA-568-B.2-1 compliant) | Cabling | As req. (m) |
| 13 | Electrical Power Cable (RDSO ELRS/SPEC/ELC/0019) | Cabling | As req. (m) |
| 14 | Mounting Enclosure / Housing for Electronics — IP68 (roof-mounted external components: Camera Unit, Antenna housings); IP54 minimum (under-roof internal cabinet components) | Enclosure | 1 |
| 15 | Cable Connectors, Glands, and Accessories | Accessories | 1 set |

## jD.2 Central / Server Side BOM (MSD should be minimised)

| S.No. | Item Description | Sub-system | Qty |
| --- | --- | --- | --- |
| 1 | Central Web Server (Application + Database) | Central Server | 1 |
| 2 | MeghRaj / CRIS Approved Cloud Storage Platform | Cloud Infrastructure | 1 instance |
| 3 | Network Attached Storage / Cloud Backup Server | Central Storage | 1 |
| 4 | Dashboard Web Application Server (HMI) | Dashboard / HMI | 1 |
| 5 | Network Switch / Router (Central Site) | Networking | 1 |
| 6 | UPS / Power Backup (Central Server Room) | Power Supply | 1 |
| 7 | Workstation / Client Terminal for Administrators | Client Access | As req. |
| 8 | SMS Gateway System (for alert delivery) | Alarm System | 1 |
| 9 | Network Security Appliance / Firewall (Central Server) | Cybersecurity | 1 |
| 10 | Centralised Monitoring & Network Management Software | Software | 1 set |
| 11 | All System Software Licenses (Perpetual) | Software | 1 set |
| 12 | Maintenance Toolkit & Calibration Equipment | Maintenance | 1 set |

E) POWER SUPPLY & REDUNDANCY

## E.1 Power Supply on Locomotive

The AICMS system is designed to operate using the standard auxiliary power available on Indian Railways locomotives and coaches, which provides a nominal voltage of 72V DC, 110V DC, or 110V AC. The integrated AICMS Power Supply Unit (PSU) connects directly to this locomotive distribution circuit and automatically converts the incoming voltage to the regulated levels required by each on-board hardware component. This conversion configuration is fully isolated to protect sensitive edge computing modules and cameras from transient electrical spikes and high-voltage traction line interference.

## E.2 PSU Technical Specifications

| Parameter | Specification / Requirement |
| --- | --- |
| Input Power Supply | 72V DC & 110V DC / 110V AC (nominal) — from locomotive main power distribution box |
| Safety Standard | BIS (IS-13252: Part 1: 2010 or latest) — for IT Equipment Safety |
| Installation | Power cables from system to main distribution box — in scope of supplier |
| Inbuilt / External PSU | PSU can be inbuilt or external to the system as per OEM design |
| Earthing Protection | Earthing/Surge protection arrangement to be provided by the supplier |
| Wireless Battery Life | For any wireless sensors/components: minimum 12 months battery life between changes |
| Redundancy | Adequate redundancy to ensure uninterrupted operation during locomotive operation |
| Overvoltage Protection | Built-in overvoltage and reverse polarity protection |
| EMI Shielding | PSU shielded against 25kV AC OHE electromagnetic interference |

## E.3 Cable Specifications

| Cable Type | Standard / Specification |
| --- | --- |
| Network / Data Cable | STP CAT-6 or better — Fire Retardant Shielded Twisted Pair; compliant with EIA/TIA-568-B.2-1 and IEC/EN 60332-1 (or equivalent BIS) |
| Electrical Power Cable | Compliant with RDSO specification ELRS/SPEC/ELC/0019, Rev. 3, February 2017 (or latest Revision); from RDSO-approved vendor sources |

## E.4 Redundancy & Fault Tolerance

- Network fault-tolerant recording: if network connection is unavailable, system automatically stores data and transmits to server on connection restoration.

- On-board NAS provides minimum 1-month local data backup accessible remotely — preventing data loss during network outages.

- Self-diagnostic system generates alarms to Indian Railways in real time for any system failure, breakdown, theft, or vandalism.

- Power supply designed for fail-safe operation — system should not affect normal working of OHE components, Signalling, Telecom, or any other Indian Railways working system.

F) OPERATION & MAINTENANCE OVERVIEW

## F.1 System Operation

The AICMS system is designed for fully automatic, unattended operation during locomotive service. No operator intervention is required for defect detection. The system activates automatically when the locomotive is operational and begins monitoring the OHE continuously throughout the journey.

## F.2 Operational Features

- Autonomous Monitoring: Fully automated defect detection operating continuously without requiring any human intervention or manual monitoring.

- All-Weather Capability: Engineered for 24×7, all-weather operations across all climatic conditions and lighting environments encountered on the Indian Railways network.

- Modular & Portable Design: Highly portable and easily migratable architecture, designed for rapid installation and removal without the need for fixed rack-mounted hardware or extensive locomotive rewiring.

- Non-Intrusive Integration: Completely non-intrusive design ensuring zero interference with the normal working of OHE infrastructure, signaling networks, telecom assets, or any other locomotive sub-systems.

- Territory Versatility: Fully operable across single-line, multiple-line, and heavily congested electrified routes across all diverse Indian terrains.

## F.3 Maintenance Framework

| Maintenance Type | Description |
| --- | --- |
| Preventive Maintenance | Periodic health check of camera, sensors, communication module, and PSU as per OEM schedule |
| Corrective Maintenance | Fault diagnosis and rectification including remote diagnostics and on-site support |
| AI Model Retraining | Retraining and updating of AI/ML models based on new OHE failure data and field experience |
| Software Updates | Periodic software patches, security updates, and feature upgrades |
| Calibration | System to be maintenance-free during warranty — optics and sensors designed for no manual cleaning/recalibration |
| Spare Parts | OEM to furnish documentary proof of hardware/firmware/software upgrade capability and spares availability for minimum 6 years |

## F.4 DLP (Defects Liability Period) Obligations

- Supplier to depute O&M personnel for the entire Defects Liability Period (DLP).

- Deployed personnel to analyse, validate, and perform requisite operations on dynamically detected defects to confirm if real or false detections.

- After successful completion of DLP, supplier to hand over activity to end customer/operator with proper Training & Documentation.

## F.5 Warranty

- Supplier to confirm warranty of complete AICMS system for a minimum period as mentioned in the tender specification.

- Warranty support to include remote/on-site diagnostics, periodic software updates, and retraining of AI models based on new OHE failure data.

- System to be maintenance-free during warranty period with optics and sensors designed to operate without manual cleaning or recalibration.

G) SYSTEM TRIALS & ACCEPTANCE

## G.1 Trial Scheme

During field trials, the AI model validation is benchmarked using known OHE defect scenarios encountered on Indian Railways. The core machine learning algorithms adaptively improve detection accuracy over successive trial runs. The exact field trial framework and schedules are finalized under the direction of the PCEE of the controlling Zonal Railway.

| Trial Phase | Description | Duration / Condition |
| --- | --- | --- |
| Factory Acceptance Test (FAT) | Validation of system components, software, hardware, and interfaces against this Design Document and TI/IN/0057 | Prior to despatch |
| Learning / Training Trial | AI model trained and validated on actual Indian Railway OHE sections with real defect scenarios | Within 2 months of installation |
| Field Trial | Complete system validated in actual operational conditions on designated loco(s) | As per PCEE schedule |
| Site Acceptance Test (SAT) | Final validation and acceptance after successful field trial. System declared fit for actual monitoring. | Post field trial |

## G.2 Acceptance Criteria

- System detects all primary events listed in Clause 4.0 of TI/IN/0057 without fail at line speeds up to 160 kmph.

- Defect-to-server communication latency: average < 20 seconds; 95th percentile < 90 seconds depending on the network.

- False alarm rate: minimal — any limitations to be declared by OEM in advance.

- Self-diagnosis and failure alarm system operational and verified.

- Data retention: local NAS minimum 1 month; server minimum 1 year — verified.

- All applicable BIS/EN/IEC test certificates submitted and verified by accredited labs.

- Cybersecurity compliance (TI/IN/0052(02/25)) certificate submitted.

- OEM Declaration Certificate and NDA submitted as per TI/IN/0057 Clause 9.3.

H) CONCLUSION

The Artificial Intelligence Enabled Loco Based Condition Monitoring System (AICMS) represents a major step forward for Indian Railways in transitioning from traditional, periodic OHE inspections to proactive, data-driven predictive maintenance. By deploying localized AI-powered edge analytics on electric locomotives built at Patiala Locomotive Works (PLW), Indian Railways will significantly improve real-time fault detection and asset reliability.

The architecture developed by Ductus Infodot Pvt. Ltd. fully satisfies all technical, functional, environmental, and cybersecurity mandates specified under RDSO Instruction No. TI/IN/0057. It provides a portable, non-intrusive, and highly scalable solution for continuous OHE monitoring across all electrified routes.

### Core Operational Benefits:

- Enhanced Safety: Drastic reduction in OHE failures, traffic disruptions, and traction-related accidents through immediate automated defect detection.

- Real-Time Response: Instantaneous transmission of precise, GPS-tagged alerts and defect imagery directly to Zonal and Divisional control centers.

- Data-Driven Insights: Creation of a secure, comprehensive historical data repository to support predictive trend analysis and evidence-based asset management.

- Optimized Resources: Minimization of high-frequency manual foot patrolling and scheduled tower car inspections, allowing technical teams to focus on targeted maintenance activities.

- CRIS/TDMS Integration: Open API and data architecture engineered for native, seamless integration with existing railway software systems and the CRIS cloud.

- Absolute Sovereignty: Full alignment with Government of India data localization rules, ensuring all operational feeds remain exclusively the property of Indian Railways.

This Design Document serves as the foundational technical reference governing the procurement, rapid deployment, automated testing, and long-term operational framework of the AICMS configuration for Patiala Locomotive Works and the wider Indian Railways network.

— End of Design Document for AICMS System —